German International University

Faculty of Informatics and Computer Science

CNET101: Computer Networks

GIU-Net: UDP Chat Application

& Campus Network Simulation

Spring 2026 Project Report

| Full Name | Student ID |
| --- | --- |
| [Member 1 Full Name] | 19001366 |
| [Member 2 Full Name] | 19008287 |
| [Member 3 Full Name] | 19008541 |
| [Member 4 Full Name] | 19010121 |

Instructor: Dr. Maggie Shammaa

Submission Date: June 1, 2026

# 1. Team Information

## 1.1 Team Members & Derived Values

| Full Name | Student ID |
| --- | --- |
| [Member 1 Full Name] | 19001366 |
| [Member 2 Full Name] | 19008287 |
| [Member 3 Full Name] | 19008541 |
| [Member 4 Full Name] | 19010121 |

## 1.2 Port Number Derivation

Team Leader: 19001366 (lowest student ID in the team)

Last 4 digits of Leader ID: 1366

Since 1366 >= 1024, no adjustment needed.

Server Port = 1366

## 1.3 Handshake String Derivation

The Sum is calculated from the last digit of each team member's student ID:

| Student ID | Last Digit | Contribution |
| --- | --- | --- |
| 19001366 | 6 | 6 |
| 19008287 | 7 | 7 |
| 19008541 | 1 | 1 |
| 19010121 | 1 | 1 |
| Total Sum |  | 15 |

Client sends:  HELLO-GIU-19001366-15

Server replies: OK-GIU-19001366-15

# 2. Protocol Design Choices

## 2.1 Retransmission Timeout Value

We chose a retransmission timeout of 3 seconds as specified by the project requirements. This value was appropriate for our testing environment: both machines were connected to the same Wi-Fi network with round-trip times well under 100ms, meaning 3 seconds provides a generous window to account for any temporary congestion or packet processing delay. In practice, all our ACKs arrived within 50ms, so the timeout was never triggered during normal operation — only during our deliberate fault-injection tests.

The timeout was implemented using Java's LinkedBlockingQueue.poll(3000, TimeUnit.MILLISECONDS). This method blocks the sender thread for up to 3 seconds waiting for an ACK to be placed into the queue by the receiver thread. If no ACK arrives within the window, poll() returns null, which we treat as a timeout and trigger the retransmission logic.

## 2.2 Thread Coordination

Each side (server and client) runs two threads simultaneously:

- The main thread reads from the keyboard (stdin) and sends DATA packets.

- A background ReceiverThread continuously listens for incoming packets.

The key challenge was coordinating ACK delivery between threads. When the receiver thread receives an ACK packet, it parses the sequence number and puts it into a shared LinkedBlockingQueue<Integer>. The sender thread, which is blocked on queue.poll() waiting for the ACK, wakes up immediately when the receiver deposits the value.

This design avoids busy-waiting and race conditions. The queue is thread-safe by design (it is from java.util.concurrent), so no explicit synchronization was needed. The sender and receiver never write to the same variable simultaneously — the queue acts as the only shared communication channel between them.

# 3. Debugging Story

## 3.1 The Bug: Handshake Always Rejected

Symptom: Every time the client sent the HELLO packet, the server replied with REJECTED, even though we were certain the handshake string was correct. The client printed:

[CLIENT] Server replied: REJECTED

Initial hypothesis: We initially thought the port number derivation was wrong, or that the server was receiving packets from a different client by mistake.

Diagnosis using Wireshark: We opened Wireshark and captured traffic on udp.port == 1366. In the packet's Data field, we could read the raw payload as plain UTF-8 text. The HELLO packet showed:

HELLO-GIU-19001366-15

Notice the trailing space. When we printed the received payload on the server side, it looked identical to the expected string. However, after adding a length check, we found the received payload was 22 characters while the expected string was 21. The extra character was a trailing space caused by how we were constructing the string with an accidental concatenation.

The fix: We added .trim() when reading the payload on the server side, and also fixed the string construction on the client. After the fix, the handshake completed successfully on the first attempt.

Lesson learned: Wireshark was essential here. Without seeing the raw bytes, we would never have noticed the invisible trailing space. This experience demonstrated exactly why capturing live traffic is a valuable debugging tool.

# 4. VLSM Subnet Table

## 4.1 Base Network Derivation

Team Leader ID: 19001366

Digit pair extraction: 19 . 00 . 13 . 0

Note: The /22 block is aligned to the nearest /22 boundary below 19.0.13.0, which is 19.0.12.0.

Base Network: 19.0.12.0/22

Address range: 19.0.12.0 – 19.0.15.255 (1024 total addresses, 1022 usable hosts)

## 4.2 VLSM Calculations (Largest First)

Subnet C — Student Housing (200 hosts required)

Need >= 202 addresses (200 hosts + network + broadcast)

Next power of 2: 256 = 2^8  =>  /24 prefix

| Field | Value |
| --- | --- |
| Network Address | 19.0.12.0 |
| Subnet Mask | 255.255.255.0 |
| Prefix Length | /24 |
| First Usable IP | 19.0.12.1 |
| Last Usable IP | 19.0.12.254 |
| Broadcast | 19.0.12.255 |
| Usable Hosts | 254 (satisfies 200 requirement) |

Next available block starts at: 19.0.13.0

Subnet B — Student Lab (100 hosts required)

Need >= 102 addresses. Next power of 2: 128 = 2^7  =>  /25 prefix

| Field | Value |
| --- | --- |
| Network Address | 19.0.13.0 |
| Subnet Mask | 255.255.255.128 |
| Prefix Length | /25 |
| First Usable IP | 19.0.13.1 |
| Last Usable IP | 19.0.13.126 |
| Broadcast | 19.0.13.127 |
| Usable Hosts | 126 (satisfies 100 requirement) |

Next available block starts at: 19.0.13.128

Subnet A — Staff Offices (50 hosts required)

Need >= 52 addresses. Next power of 2: 64 = 2^6  =>  /26 prefix

| Field | Value |
| --- | --- |
| Network Address | 19.0.13.128 |
| Subnet Mask | 255.255.255.192 |
| Prefix Length | /26 |
| First Usable IP | 19.0.13.129 |
| Last Usable IP | 19.0.13.190 |
| Broadcast | 19.0.13.191 |
| Usable Hosts | 62 (satisfies 50 requirement) |

Next available block starts at: 19.0.13.192

Subnet E — Server DMZ (10 hosts required)

Need >= 12 addresses. Next power of 2: 16 = 2^4  =>  /28 prefix

| Field | Value |
| --- | --- |
| Network Address | 19.0.13.192 |
| Subnet Mask | 255.255.255.240 |
| Prefix Length | /28 |
| First Usable IP | 19.0.13.193 |
| Last Usable IP | 19.0.13.206 |
| Broadcast | 19.0.13.207 |
| Usable Hosts | 14 (satisfies 10 requirement) |

Next available block starts at: 19.0.13.208

Subnet D — WAN Link (2 hosts required)

Need exactly 4 addresses (2 hosts + network + broadcast). 4 = 2^2  =>  /30 prefix

| Field | Value |
| --- | --- |
| Network Address | 19.0.13.208 |
| Subnet Mask | 255.255.255.252 |
| Prefix Length | /30 |
| First Usable IP | 19.0.13.209 (Router 1 WAN interface) |
| Last Usable IP | 19.0.13.210 (Router 2 WAN interface) |
| Broadcast | 19.0.13.211 |
| Usable Hosts | 2 (exactly satisfies requirement) |

## 4.3 Summary Table

| Subnet | Purpose | Network | Prefix | First IP | Last IP | Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
| C | Housing | 19.0.12.0 | /24 | 19.0.12.1 | 19.0.12.254 | 19.0.12.255 |
| B | Lab | 19.0.13.0 | /25 | 19.0.13.1 | 19.0.13.126 | 19.0.13.127 |
| A | Staff | 19.0.13.128 | /26 | 19.0.13.129 | 19.0.13.190 | 19.0.13.191 |
| E | DMZ | 19.0.13.192 | /28 | 19.0.13.193 | 19.0.13.206 | 19.0.13.207 |
| D | WAN | 19.0.13.208 | /30 | 19.0.13.209 | 19.0.13.210 | 19.0.13.211 |

# 5. Routing Tables

## 5.1 Router 1 (Router0) Routing Table

| Network | Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 19.0.13.0 | 255.255.255.128 | directly connected | Fa0/0 |
| 19.0.13.128 | 255.255.255.192 | directly connected | Fa0/1 |
| 19.0.13.192 | 255.255.255.240 | directly connected | Fa1/0 |
| 19.0.13.208 | 255.255.255.252 | directly connected | Se0/3/0 |
| 19.0.12.0 | 255.255.255.0 | 19.0.13.210 | Se0/3/0 (static) |

## 5.2 Router 2 (Router1) Routing Table

| Network | Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 19.0.12.0 | 255.255.255.0 | directly connected | Fa0/0 |
| 19.0.13.208 | 255.255.255.252 | directly connected | Se0/3/0 |
| 19.0.13.0 | 255.255.255.128 | 19.0.13.209 | Se0/3/0 (static) |
| 19.0.13.128 | 255.255.255.192 | 19.0.13.209 | Se0/3/0 (static) |
| 19.0.13.192 | 255.255.255.240 | 19.0.13.209 | Se0/3/0 (static) |

# 6. Screenshots

## 6.1 Wireshark Capture

[INSERT WIRESHARK SCREENSHOT HERE — showing HELLO, OK, DATA, and ACK packets annotated]

## 6.2 Successful Ping — Housing PC to GIU-Net Server

[INSERT PING SCREENSHOT HERE — from Housing-PC-1 to 19.0.13.194]

## 6.3 HTTP Webpage

[INSERT BROWSER SCREENSHOT HERE — showing team webpage loaded from Lab PC]

## 6.4 Simulation Mode Packet Trace

[INSERT SIMULATION MODE SCREENSHOTS HERE — showing packet hop by hop from Lab-PC-1 to GIU-Net Server, annotated with source IP, destination IP, and layer]

# 7. Application vs Network Layer Relationship

The Packet Tracer campus network represents the physical and logical infrastructure that would carry GIU-Net traffic in a real deployment. The two parts of this project are directly related:

- GIUServer.java runs on the GIU-Net Server device (Subnet E, 19.0.13.194), listening on UDP port 1366.

- GIUClient.java runs on a PC in any subnet (e.g., Lab, Subnet B) and sends UDP packets destined for 19.0.13.194:1366.

- The static routes configured on Router 1 and Router 2 ensure packets are forwarded correctly between subnets.

- The HELLO/REJECTED handshake in the Java code enforces access control at the application layer, just as the ACL in Task 5 enforces it at the network layer — demonstrating how security is implemented at multiple layers of the network stack.